DRAFT

Impacts of Climate Change on Coastal Infrastructures in the Caribbean

Background:

Today more than before, the impacts of climatic changes are almost a certainty considering the latest estimates published by the Intergovernmental Panel on Climate Change (IPCC, 2007). Indeed, those impacts,  which can be measured with  high level of confidence,  can already be observed  through varied geo-morphologic and hydro-climatic changes affecting  the planet’s  structure :  increasing sea level rise, melting of the artic glaciers due to global warming, and significant changes in the patters, intensity  and  frequency in return periods  of extreme weather systems,  such as  El Niño, La Nina, cyclones (hurricanes and typhoons), storm surges.

Those above-mentioned impacts, while global in scope, will however have different socio-economic consequences on the different countries of the planet, irrespective of geographic location, size, geo-physiographic characteristics, and socio-economic development patterns. However, the Small Island Development States (SIDS) are expected to be particularly affected due to their low-lying geographic location, and the concentration of their socio-economic development activities in the Coastal Zone (CZ). More than 80 % of the total population in the major cities in most of the countries of the region is concentrated in the coastal zone. As a result, most of the socio-economic infrastructure (public markets, airports, tourism amenities, roads),  and critical infrastructures hospitals, government offices and schools are also located in that area, increasing significantly the vulnerability risk level of the region to the adverse effects of climate related hazards. These vulnerability risks, while not accurately  described and quantified today,  are  nevertheless expected to increase, when value estimates  of  long term CC parameters   are integrated in the analysis, where among other changes, a yearly sea level rise of 5 mm average is to be expected.

Present and future Costs of Climate Change in the Caribbean

Globally, losses in insurance premiums payouts and Total Economic Losses due to extreme weather events have been increasing exponentially over the last 4 decades beginning in 1960, moving from US $ 10 Billion and US $ 40 Billion for the 1960-1970 decade to US $ 75 Billion and US $ 280 Billion for the 1988-1997 decade. In the Caribbean, total economic losses due to extreme weather related events, as reported for the period 1979-2005, have been estimated at a compounded value of US $ 17 Billion, which corresponds to an average annual loss of US $ 613 million. Unfortunately, those Losses are expected to increase in the near future, impacting considerably the socio-economic viability of the countries of the region. Indeed, the incremental costs of the adverse impacts of climate change on CARICOM countries over the next 50 years (though 2050) is estimated at an average of US $ 1999 million/year,  representing an estimated percentage of the region’s total GDP ranging from 5.6 to 34 percent.

While damages are expected to be wide-spread and affecting all socio-economic sectors, the costs to physical infrastructures are expected to be the most important, and would range between 990 million to 6.600 million over the next 50 years, making the costs of the impacts of Climate change on that specific sector the costliest that the region will be facing in the relative medium-term.  As a result, identifying approaches and mechanisms for addressing the present and future impacts of CC on the region’s infrastructures is of primary importance. Furthermore, considering the high level of dependency of a country’s socio-economic development and functioning to the availability of reliable infrastructures, especially in post-disaster contexts, reducing the vulnerability risk of that specific sector will ultimately contribute in reducing the impacts of climate related hazards to the overall socio-economic development of a country.

Addressing the impacts of Climate Change on the SIDS: Justification

While the region’s contribution to global emission of Green House Gazes (GHG) is negligible, - less than 6 percent (%) of the total global emission, - the changes in the planet’s climate will have far more consequences on the Caribbean Region, threatening irreversibly the sustainable livelihood and  the socio-economic foundations of the region as a whole. An additional concern is the fact that the small size of the many Caribbean countries limits their development options and thus their ability to avoid, withstand, or recover from extreme weather events such hurricanes, storms and floods Nevertheless, the only alternatives for the region remain the integration at the national and regional levels of adaptation and mitigation mechanisms and approaches capable of reducing the vulnerability risk associated with climate change. And, considering that the bulk of the costs will be borne by the built environment within the coastal zone, designing and implementing adaptive and mitigative approaches and methods to reduce the projected impacts on that specific sector will constitute an important step towards providing meaningful long term support to the region in addressing the adverse impacts of climate change.

Addressing the Impacts of Climate Change on the SIDS’ Coastal Infrastructures

Today, while fairly accurate estimates of the level of damages and resulting costs associated with the adverse impacts of Climate-related extreme events on the built environment can be generated, comprehensive mitigation and adaptive measures still need to be formulated for the region. The techniques and approaches to achieve that goal exist, but need to be adapted to suit the geographic, economic and physical circumstances in SIDS. More accurate data need to be generated for the region, considering that the level of impacts is estimated mostly through extrapolating hemispheric/ global climate damage assessment models. Therefore, there is a clear need for region-specific risk damage assessment models capable of providing more accurate estimates, which in turn will better inform the region’s national climate change adaptation policies and strategies, and most importantly provide a greater level of confidence in the selection and implementation of vulnerability risk assessments methods.

From an inferential statistical analysis standpoint, the issue above-mentioned associated with the level of confidence of the vulnerability assessment estimates as presently generated through global CC models for the region,  do not in fact  provide the best indicators of the real level of damages to be expected, when compared to the level of losses that one could generate through in situ post-disaster damage assessments or in situ vulnerability assessments – Thus it is  difficult to grasp the real level of damages to be expected and hence formulating fairly accurate adaptation  and mitigation policies and planning interventions . Therein lies the need for down-scaling the existing climate models (global/hemispheric to regional) so that the different variables of the main climate change parameters: (winds speed and trajectory; sea level rise) be determined specifically for the region. This should lead to a re-evaluation of the varied parameters describing the region’s real exposures to the adverse impacts of climate changes.

Vulnerability Assessment (VA) of the Coastal Infrastructures of the SIDS to the Adverse Impacts of Climate Change

Project Objective:

This  project  aims at :  (1) assessing the vulnerability risks associated with the adverse Impacts of Climate Change on the Integrity of Coastal Critical Infrastructures,  such as  Hospital, shelters, fire stations and storm Barriers;  (2) strengthening the resilience of  the infrastructures with the highest risk profile through the design and implementation of mitigative and adaptive mechanisms; (3) describing the macro-economical impacts on national economy of the CC impacts, as a results of the impacts on coastal critical infrastructures; and (4) strengthening the technical  capacities of the building professionals (physical planners and engineering officials) in conducting vulnerability analyses.

It is intended to be undertaken in the coastal zone of Kingston the capital city of Jamaica, and will be built on the similar interventions being realized in the Castries waterfront of St Lucia by the World Bank and the Climate Change Center, through the Implementation of Adaptation Measures in Coastal Zones (SPACC) project.

Project main Expected Outcomes:

The main outcome of this project  is to reducing the vulnerability risks of the Caribbean Smalls Islands Coastal infrastructures to the adverse impacts of CC, through accurately estimating the risks for the region to CC’s impacts, and proposing and applying appropriate mitigative and adaptive  measures.

Project expected Outputs:

In term of outputs, we expect by the end of this project that:

- Parameters describing and estimating -within acceptable intervals of confidence- the vulnerabilities and risks of coastal infrastructures to climate related hazards are determined;

- Macro-economical (analysis) parameters describing the socio-economical impacts on national economy of the adverse effects of CC – as a result of its expected impacts on Coastal Infrastructures- determined;

- Region specific Mitigative and adaptative measures capable of effectively reducing the vulnerabilities to Climate related hazards of the coastal physical infrastructures are formulated;

- Using multi-disciplinary and cost-benefits analysis considerations,  the most urgent coastal infrastructure to be retrofitted in the immediate  is identified, and sub-consequently retrofitted; and

- Final Results of the project initiative successfully published in a well-known scientific/engineering physical planning magazine.

Project’s main proposed Activities:

The main activities to be undertaken under this initiative are to:

- Assess the particular vulnerability and risks of coastal (critical) infrastructures to climate related natural hazards (hurricane and Storm surge, tsunamis) - and the corresponding cost-benefit estimates  for the  adaptive and mitigating measures being proposed ;

- Demonstrate the increasing vulnerably and risks to the region when climate change (global warming parameters) parameters are being considered and integrated  into  the assessment methods;

- Identify the specific engineering and scientific  techniques and approaches capable of reducing the level of vulnerabilities- as assessed in 1 and 2- , and propose Mitigation measures for reinforcing  the identified critical infrastructures (Hospital, shelters, fire stations, and storms Barriers);

- Select – based on the most likelihood of structural failure (as assessed and demonstrated in 1, 2 and 3) and in situ damages costs estimates in human lives and  financial losses, among others – one (1) critical infrastructure to be retrofitted; and

- Publish in collaboration with the 5C and the OAS/DSD the results of the studies – This is to be realized in a well-known engineering/ physical planning/ scientific magazine – so that it can sustain peer review.

From a technical standpoint, the activities to be undertaken will be within the expertise of the fields of physical planning studies, engineering, architectural considerations, and macro-economy. More specifically, it will imply:

- Climate Hazards Risks Studies   (climate systems):

Storm surge hazard assessment – Wave Hazard Assessment – Torrential rain Analysis – and Wind Hazard Assessment;

- Critical Infrastructures’ vulnerability assessment and cost-estimates (built environment):

Inventory of Critical infrastructures and critical buildings - Vulnerability analyses of Critical built infrastructure and critical buildings - risks assessment of critical built infrastructure and critical buildings – multi-disciplinary decision-making for selecting the infrastructure /building to be retrofitted- and design Stage I for the Retrofitting project;  and

- Macro-economic Analysis  and Cost benefits Estimates (cost-benefit analysis):

Costs estimates of repairs and reconstruction of critical built infrastructure – cost saving to national economy.

Project estimated Timeframe:    24 months

Project TOTAL Estimated Costs: Order of US $ Millions – TBD Following multi-disciplinary team’s Inception visits and Reports.

Technical Rational

Adaptation and mitigation as pro-active and coping mechanisms required the availability of fairly accurate data (estimates and indicators) on the risks to which the costal built environment is being exposed, which risks are mostly related to the combined effects of winds and sea waters (Hurricanes, storm surges, tsunamis) and to some extent, underwater and ground-shaking for earthquakes and volcanoes having the potential to leading to tsunamis.

As a result, the most important inputs to be generated are accurate data estimates of winds loads as applied for the Caribbean Region, which requires the availability of updated wind hazards maps for the region. These maps could later on be used for undertaking VA in other countries of the Caribbean Basin region, as the regional wind loads inputs generated would be the same.

- Caribbean Winds Hazard Maps

The only Pan-Caribbean wind hazard maps ever produced for application in the design of structures and for estimating the failure of as a result,  were produced in 1969 by the Caribbean Meteorological Institute, followed in 1981 by the updated maps generated by the Caribbean Meteorological Institute, and the latest created in 1985, were the wind hazard maps published by the University of Western Ontario (UWO) using updated data accumulated from test results generated through their Boundary Layer Wind Tunnel Laboratory.  Since 1985 the region has had the opportunity to collect another 20 years of relatively reliable raw data on wind/climate related events, while experiencing/ witnessing new patterns in wind and climate phenomena - resulting from climate variability. As a result, any initiative aiming  at addressing the vulnerability risk to climate change requires the updating of those maps, which maps are fundamental in determining accurately the level of risks  and hence the vulnerabilities  to be reduced.

Technical, institutional and Managerial Arrangements

A technical team composed of experienced civil Engineers specialized in impacts assessments for the built environment and climate related scientists will be in charge of conducting the project activities. This will include but not limited to:

- The Boundary Layer Wind Tunnel Laboratory (BLWTL) – of the Alen G Davenport Wind Engineering Group,  at the University of Western Ontario (UWO) in London Ontario, Canada;

- The Department of Civil and Coastal Engineering at the University of Florida’s College of Engineering,  in Gainesville Florida, USA;

- The Wind Load Test Facilities at the Department of Civil Engineering  at Clamson University, in South Carolina, USA;

- The CARICOM Climate Change Center (5C);  and

- The Faculty of Engineering at the University of the West Indies (UWI), St Augustine, Trinidad and Tobago;

The multi-disciplinary technical team will be – preferably - coordinated by a renown engineer with background in Civil Engineering and/or Scientist with good understanding of the engineering processes that the project implies– who has a strong working relationship with all the entities in  the team– so that to facilitate the coordination and integration of all the different entities, and  would also be responsible of coordinating the peer reviewing and publication of the technical/engineering project’s results. More specifically the following specialists are expected to be part of the team: Climate Scientists, Coastal Engineers, Meteorologists, Physical Planners, Macro-economist, Civil Engineers, and Structural Engineers with particular expertise in Wind and Flood Resistant Design, architects, and construction contractors.

Overall Management and Coordination will be executed by the OAS/DSD.

Project Activities – Proposed Timeline

- Activities to be realized in the Short-term:  0-6 months

In collaboration with the project team leader (to be identified and selected, and TOR to be drafted), detail work plans for the project will be produced:

- Scientific and Engineering Inception Studies: Identify the scope and depth of all the  necessary engineering studies to be undertaken in order to achieve the activities  above listed;

- Budget and Workplan : Provide comprehensive cost estimates for undertaking such works (as above outline);

- Technical and Institutional Arrangements: Establish the roles and responsibilities of the engineering and scientific team that will be responsible of executing the activities- along with clear identification of the credentials and technical responsibilities of each team member (institutions); and

- Proposed Timeframe with milestones to be achieved/ unit of time.

TOTAL estimated COSTS:  US $ 800,000 (see Table I for costs breakdowns)

    - Implementation Activities - Mid/long terms: 6-24 months

The main activities to be undertaken under this initiative are to:

- Assess the particular Vulnerability and risks of coastal (critical) infrastructures to climate related natural hazards (hurricane and Storm surge, tsunamis) - and the corresponding macro-economical consequences on national economy and cost-benefit estimates  for the corresponding  adaptive and mitigating measures;

- Demonstrate the increasing vulnerably and risks to the region when climate change (global warming parameters) parameters are being considered and integrated  into  the assessment methods;

- Identify the specific engineering and scientific  techniques and approaches capable of reducing the level of vulnerabilities- as assessed in 1 and 2- , and propose Mitigation measures for reinforcing  the identified critical infrastructures (Hospital, shelters, fire stations, and storms Barriers);

- Select – based on the most likelihood of structural failure (as assessed and demonstrated in 1, 2 and 3) & damages costs in human lives and financial, among others – one (1) critical infrastructure to be retrofitted;

- Publish in collaboration with the 5C and the OAS/DSD the results of the studies – This is to be realized  in a well-known engineering/ physical planning/ scientific magazine – so that it can sustain peer review; and

Project Implementation Cost Estimates

Will be in the Order of US $ Millions – TBD Following multi-disciplinary team’s Inception visits and Reports

| Table I - Impacts of Climate Change on Coastal Infrastructures in the Caribbean |  |  |
| --- | --- | --- |
| Inception Activities Costs Breakdown |  |  |
| Activity # | Components | Costs (in $ US) |
| 1 | Scientific and Engineering Inception Studies | 500,000.00 |
| 2 | Budget and Work plan | 200,000.00 |
| 3 | Technical and Institutional Arrangements | 100,000.00 |
| TOTAL |  | 800,000.00 |

OAS/DSD -  Caribbean Division

Level of confidence used in the report is  90 %  -    ± 5 % uncertainty (IPCC, 2007)

Average rate of sea level rise estimate at 3.1 mm for the last 10 decade, while the total increased for the whole 20th Century is 0.17m (IPCC, 2007)

Refers to the Interface air, or, in geological terms, the lithosphere, hydrosphere, and atmosphere join to form triple interface.”… hence includes the Exclusive Economic Zone (EEZ), (Peel Brahtz, 1972, 2)

Global surface air  warming over the 21st century is expected to increase en average from 1.8o to 4.0oC – for a see level rise ranging from 0.18 to 0.59 m (IPCC, 2007)

CRID, UNEP.

WB, 2007.

WB unpublished report

WB unpublished report

Region here included Caribbean and Latin America – 8.5 % of planet’s population (IEA, 2005;  WRI, 2006)

Corals reefs in the Caribbean are believed to have been irreversibly damages  since the 1980s (A. Strong, 2006)

The Pan-American Health Organization (PAHO) is embarked on an initiative from which the Wind Hazards Maps (necessary fro undertaking that pilot project) will be produced and might be made available by the time the project start.

H C Shellard, 1969

Rocheford, 1981

Davenport, Surry, Georgiou, 1985